TUTORIAL-7

NAME: Yadnesh Dhomble     ROLL NO:16       PRN:12412333       IP DIV-1    IP BATCH-2

Edge detection and morphological operations such as dilation and erosion

CODE:

clc;

clear;

close all;

% Read image

img = imread('my_image.jpg');

% Convert to grayscale manually

gray = double(img(:,:,1))*0.2989 + double(img(:,:,2))*0.5870 + double(img(:,:,3))*0.1140;

[m,n] = size(gray);

% Sobel edge detection kernels

Gx = [-1 0 1; -2 0 2; -1 0 1];

Gy = [-1 -2 -1; 0 0 0; 1 2 1];

edge_img = zeros(m,n);

% Edge Detection

for i = 2:m-1

for j = 2:n-1

region = gray(i-1:i+1 , j-1:j+1);

sx = sum(sum(region .* Gx));

sy = sum(sum(region .* Gy));

edge_img(i,j) = sqrt(sx^2 + sy^2);

end

end

edge_img = edge_img / max(edge_img(:));

% Structuring element for morphology

SE = [1 1 1; 1 1 1; 1 1 1];

dilation = zeros(m,n);

erosion = zeros(m,n);

% Dilation

for i = 2:m-1

for j = 2:n-1

region = edge_img(i-1:i+1 , j-1:j+1);

if any(region(SE==1))

dilation(i,j) = 1;

end

end

end

% Erosion

for i = 2:m-1

for j = 2:n-1

region = edge_img(i-1:i+1 , j-1:j+1);

if all(region(SE==1))

erosion(i,j) = 1;

end

end

end

% Display results

figure;

subplot(2,2,1);

imshow(uint8(gray));

title('Grayscale Image');

subplot(2,2,2);

imshow(edge_img);

title('Edge Detection');

subplot(2,2,3);

imshow(dilation);

title('Dilation');

subplot(2,2,4);

imshow(erosion);

title('Erosion');

OUTPUT: