Name: Shourya Desai               AIDS-B                 Batch 2                        Roll No. 4

ASSIGNMENT 6

Write a program to implement error detection and correction for 7-bit and 8-bit ASCII encoded data using the Cyclic Redundancy Check (CRC) technique

CODE:

def xor(a, b):

result = ""

for i in range(1, len(b)):

if a[i] == b[i]:

result += "0"

else:

result += "1"

return result

def mod2div(dividend, divisor):

pick = len(divisor)

tmp = dividend[0:pick]

while pick < len(dividend):

if tmp[0] == '1':

tmp = xor(divisor, tmp) + dividend[pick]

else:

tmp = xor('0'*pick, tmp) + dividend[pick]

pick += 1

if tmp[0] == '1':

tmp = xor(divisor, tmp)

else:

tmp = xor('0'*pick, tmp)

return tmp

def encode_data(data, key):

l_key = len(key)

appended_data = data + '0'*(l_key-1)

remainder = mod2div(appended_data, key)

codeword = data + remainder

return codeword

def ascii_to_binary(text, bits):

binary = ""

for char in text:

if bits == 7:

binary += format(ord(char), '07b')

else:

binary += format(ord(char), '08b')

return binary

text = input("Enter text to transmit: ")

ascii_type = int(input("Enter ASCII type (7 or 8): "))

generator = input("Enter Generator Polynomial (binary, e.g., 1101): ")

data = ascii_to_binary(text, ascii_type)

print("\nBinary Data:", data)

codeword = encode_data(data, generator)

print("Transmitted Codeword:", codeword)

received = input("\nEnter received codeword: ")

remainder = mod2div(received, generator)

if '1' in remainder:

print("Error detected in received data!")

else:

print("No error detected.")

OUTPUT: